INDOOR LOCALIZATION FOR MECANUM-WHEELED MOBILE ROBOT: A VISION-BASED LOCALIZATION FRAMEWORK

(Định vị trong nhà cho robot di động sử dụng bánh xe Mecanum: Khung định vị dựa trên thị giác máy tính)

Trinh Dang Bao Duc2,3_2113239_Control Engineering and Automation, Bach Ngoc Phu2,3 _2114409_Control Engineering and Automation, Bui Minh Duc2,3 _2110130_Control Engineering and Automation, Nguyen Trong Tai1,3, *

1 Faculty of Electrical and Electronics Engineering, Ho Chi Minh City University of Technology (HCMUT), 268 Ly Thuong Kiet Street, District 10, Ho Chi Minh City, Vietnam

2 Faculty of Electrical and Electronics Engineering - Undergraduate Student, Ho Chi Minh City University of Technology (HCMUT), 268 Ly Thuong Kiet Street, District 10, Ho Chi Minh City, Vietnam

3 Vietnam National University Ho Chi Minh City, Linh Trung Ward, Thu Duc District, Ho Chi Minh City, Vietnam

* Corresponding author: nttai@hcmut.edu.vn

Abstract

This paper addresses the problem of indoor localization and autonomous navigation for a Mecanum-wheeled mobile robot operating in environments without LiDAR sensing. A vision-based localization framework is developed by fusing information from ArUco marker detections, inertial measurement units, and wheel odometry. Two estimate methods - a regular Extended Kalman Filter (EKF) with χ²-based statistical testing for measurement outlier rejection and an Adaptive Extended Kalman Filter (AEKF) with online noise covariance adaptation - are used and compared methodically. A nonlinear Backstepping controller is created to allow for accurate trajectory tracking while taking into consideration the particular kinematic limitations of the Mecanum platform. The complete system is experimentally validated in a structured indoor environment, demonstrating the comparative performance of the localization algorithms and the effectiveness of the control strategy. The proposed approach provides a low-cost, reliable solution for LiDAR-free indoor navigation of omnidirectional mobile robots.

Keywords: indoor localization, multi-sensor fusion, ArUco markers, Adaptive Extended Kalman Filter, χ²-testing, mecanum robot, nonlinear control, autonomous navigation

- Introduction

The use of autonomous mobile robots in indoor environments is expanding, with small-scale platforms playing a key role in prototyping algorithms for larger systems. This paper presents an integrated sensor fusion system for an ADAS model vehicle, offering a comprehensive platform to test autonomous driving algorithms before real-world deployment.

While GPS is widely used for outdoor localization, it is ineffective in constrained indoor environments, necessitating specialized indoor solutions. Accurate localization is essential for autonomous systems, as modules like motion planning and control depend on precise positional data. LiDAR (Light Detection and Ranging) enables high-precision scanning, making it suitable for mapping and relative localization in reflective indoor settings. Platforms such as TurtleBot3 and Clearpath Jackal are equipped with LiDAR sensors to support SLAM and navigation tasks [1]. However, LiDAR systems are often costly and power-intensive,

which limits their suitability for small-scale robots or budget-constrained applications.

Another approach involves acoustic-based indoor positioning systems, where robots determine their position using ultrasonic signals emitted from beacons installed in the environment. Such systems have been implemented in service robots like Keenon T1 and Savioke Relay [2]. Nevertheless, these methods are often susceptible to environmental noise, resonance, and obstructions, which degrade localization accuracy and stability.

Single-modality localization methods have inherent limitations, making multi-sensor fusion essential. Integrating vision, IMU, and wheel odometry leverages the strengths of each sensor, with fusion algorithms like EKF, AEKF, and χ²-enhanced EKF showing strong performance indoors. This study proposes and evaluates a vision-based localization system using ArUco markers fused with IMU and encoder data to provide accurate, robust state estimation for an indoor ADAS model vehicle.

- Methods and Implementation

2.1 Sensor setup and physical parameters of model robot

Table 1. Physical parameters of the mobile robot

| Parameter | Value | Meaning |
| --- | --- | --- |
| m | 6 | Mass of Robot |
| Iα | 0.24328 kg.m2 | Moment of inertia around z axis |
| r | 0.035 m | Radius of robot wheel |
| L1 | 0.11 m | Distance from robot center to wheel along vertical axis |
| L2 | 0.10 m | Distance from robot center to wheel along horizontal axis |

Figure 1. Hardware setup of the Mecanum Robot

Figure 2. Scheme design of the indoor autonomous driving framework Robot

Our indoor localization system is implemented on a custom Mecanum-wheeled mobile robot, equipped with an onboard IPC for centralized processing, a 1080p RGB camera for ArUco marker detection, and a BNO055 IMU for yaw rate sensing. Each CHR-GM37-520 motor is encoder-equipped and controlled via an Arduino ATmega 2560. Hardware specifications are listed in Tables 1, and the setup is shown in Fig. 1. The localization, trajectory planning, and control algorithms are implemented on the Mecanum-wheeled mobile robot platform described in Sec. 2.1. Fig. 1 illustrates the complete indoor autonomous navigation framework, where localization relies on three sensing sources. As outlined previously, wheel velocities and yaw rate are obtained from the encoders and the IMU, respectively, and serve as inputs for the prediction step in the sensor fusion module. The update step utilizes position and orientation measurements from ArUco markers detected by the onboard RGB camera. The fused state estimate (x, y, φ, v) is then passed to a trajectory planner.

2.2 Kinematic and dynamic model of the mobile robot system

2.2.1 Kinematic model

Consider a mobile robot with an inertial frame (xi, yi) and a body-fixed frame (xb, yb), as shown in Fig. 2. The system states include the robot's position (x, y) and orientation θ in the inertial frame, along with body-frame velocities u, v (linear) and ω (angular). Following [3], the kinematic model relates the body-frame velocities at the center point B to their counterparts in the inertial frame:

ƞ=J(θ).ζ 		        (1)

Where:

ƞ=xyθ; ζ=uvω; J(θ)=cos(θ)-sin(θ)0sin(θ)cos(θ)0001

Figure 2. Kinematic model of the Mecanum Robot

2.1.2 Dynamic model

Consider the dynamic model of a mobile robot with coordinate frames as shown in Fig. 3, where point BBB denotes the robot’s geometric center. The system parameters include mass m, dimensions l (length) and d (width), moment of inertia Icz, angular velocity ω, angular acceleration αcz​, linear accelerations acx​, acy​, and wheel diameter a. Based on [3], the corresponding dynamic equations are derived.

Figure 3. Robot dynamics model

The dynamic model of the mobile robot is developed using the Lagrangian method:

ddt(∂L∂qi)-∂L∂qi=Fi	             (2)

Here, L=KE-PE is the Lagrangian, where KE includes both translational and rotational kinetic energy, and PE = 0 as the robot moves on a flat surface. The kinetic energy is given by:

KE=0.5mxc2+yc2+0.5Iαω2		(2)

Using the velocity transformation: xc=u-ybcω; yc=v+xbcω, we get:

L=0.5mu2-2uωybc+v2+2vωxbc+ω2xbc2+ybc2+0.5Iαω2                                        		         (3)

Applying the Lagrangian formulation with generalized velocities u, v and ω, the expressions for force and torque components can be derived as shown in [3]. These expressions capture the effects of mass distribution and rotational dynamics. Based on this formulation, the general dynamic model of the mobile robot is expressed as:

Dζ+n(ζ)=τ 		         (4)

Where: ζ= u, v,ωT is the velocity vector in the body frame, D is the inertia matrix, n(ζ) represents the nonlinear terms, and τ is the vector of generalized forces and torques.

2.2 Backstepping control design

2.2.1 Kinematic and dynamic equations

The robot's system of equations is described as:

ƞ=J(ƞ).ζζ=D-1[τ-n(ζ)]		(5)

This system can be rewritten in a control-affine form as:

x=f1(x1)+g1(x1)x2x2=f2(x1,x2)+g2(x1,x2)u	       (6)

Where:

- x1=ƞ,  x2=ζ,  u =τ

- f1x1=0,  g1x1=Jƞ

- f2x1,x2=D-1-nζ,  g2(x1,x2)=D-1

- J(ƞ) is the Jacobian matrix

Let x1d=ƞd be the desired reference value, with its first and second derivatives denoted as x1d=ƞd, x1d=ƞd. Define the tracking error as e1=x1d-x1. The objective control is to ensure that e1→0 and e1→0.

2.2.2 Backstepping controller design

To design a stabilizing control law, we adopt the backstepping method as presented in [3]. Let e1=x1d-x1 and define a Lyapunov function V1= 0.5e12. Assuming e1→0, the virtual control x2d​ is selected to stabilize the first subsystem.

The second step introduces the actual control input u to stabilize the error e2=x2d-x2. A composite Lyapunov function V2=V1+ 0.5e22=0.5(e12+e22) is considered. Following the derivation in [3], the control input is chosen to ensure V2 =-k1e12-k2e22<0, with positive constants k1, k2, leading to the final control law:

u=g2-1(x, t)(k2e2+g1-1(x1)(x1d+k1e1)

+g1T(x1)e1-f2(x, t)) [4]                 (7)

This control law satisfies the Lyapunov stability condition:

u=D[k2e2+J-1(ƞ)(ƞd+k1e1)+JT(ƞ).e1]+n(ζ) (8)

2.4 ArUco marker mearsurement

AruCo is a widely adopted, low-cost fiducial marker system that can be easily integrated with various sensors for indoor robotic applications. To accurately extract visual information and estimate the robot's pose, camera calibration must be performed. This process yields two essential parameters: the intrinsic matrix of the camera and the distortion coefficients. These parameters are crucial for correcting lens distortion and enabling precise pose estimation from the detected markers.

Figure 4. PnP estimates the pose of markers relative to the camera frame

Let (XC, YC, ZC) be the 3D coordinates of marker corners in the camera frame, and (Xs, Ys) be their 2D pixel coordinates in the image. The transformation from each 3D point (xci, yci, zci) to its corresponding 2D pixel location (xsi, ysi) is described by a projection equation:

xsi  ysi  1 = fx00    0   fy 0    xs0  ys0 1 r11   r21  r31   r12r22r32   r13   r23   r33   t1   t2   t3 xci  yci  zci  1   (9)

Where i ∈{1,2,3,4} corresponds to the four corner points of the marker. The values (xs0, ys0) denote the coordinates of the principal point, and fx, fy are the scale factors in Xs and Ys axes. These four parameters are components of the camera intrinsic matrix. In addition, rij and ti represent the elements of the rotation matrix Rc and the translation vector Tc respectively. Once the Rc and Tc matrices are obtained [6], the relative orientation angle  and the distance ddd from the ArUco marker to the robot can be computed using the following equation:

φ2=arctan⁡(-r31r322+r332) 		(10)

d=(t1+llat)2+(t3+llon)2            (11)

At this point,  llat and llon represent the relative lateral and longitudinal offsets (in meters) between the camera and the robot’s center in the real-world coordinate system. Each ArUco marker is pre-defined with a known pose in the world frame, consisting of (x0, y0, 0), where x0 and y0​​ are the global coordinates and 0​ is the marker's orientation. The global position of the robot can then be estimated using the relative distance d, the relative angle 1 and the known global pose of the detected ArUco marker. The robot's orientation in the global frame, denoted as robot, is computed as: robot=φ1+φ0. During the setup phase, the camera is assumed to be rigidly mounted and aligned with the robot’s center axis. Therefore, the longitudinal offset llon is set to zero. The orientation corrections φ2 and φ are calculated as follows:

φ2=arctan⁡t1t3+llon, φ3= φrobot-φ2    (12)

Finally, the position of the robot in the global coordinate frame, expressed in meters, can be computed as:

xrobot =x0-dcos(φ3), yrobot =y0-dsin(φ3) (13)

Figure 5. Single-track model with ArUco-based pose detection in the global frame

Since the acquired image frames are often affected by noise due to lighting conditions, color variations, and other environmental factors, the estimated pose values may become unstable or inaccurate. To enhance the robustness and precision of pose estimation, we propose applying additional filtering techniques. Specifically, a Low-Pass Filter (LPF) [7] is employed with a smoothing factor  = 0.05, applied uniformly to the x, y positions and the orientation angle .

In scenarios where the robot fails to detect ArUco markers, a dead-reckoning method is used to estimate its pose based on IMU and encoder data. The velocities Vxand Vy are derived from the encoders, while the angular velocity ωt comes from the IMU. The robot's pose at time t+Δt is updated as follows:

xt+∆t= xt +(Vx.cos(t) -  Vy.sin(t)). ∆t ,

yt+∆t= yt +(Vx.sin(t) -  Vy.cos(t)). ∆t ,

t+∆t= t+ωt. ∆t.

This ensures position estimation continues even without visual marker input.

  - Estimation methods

2.5.1 EKF

Figure 6. Block diagram of EKF family with Adaptation and χ²-based Filtering

The goal of the EKF is to estimate the system's state from noisy control inputs and measurements, minimizing the estimation error. Each time step t, the EKF uses a dynamic model to predict the system's new state and the corresponding covariance matrix.

xt|t-1=fxt-1,ut, Pt|t-1=FtPt-1FtT+Q   (14)

Where fxt-1,ut is the nonlinear system model, which in this study is a kinematic single-track model [8]; Qt is the process noise covariance matrix; Ft is the Jacobian of fxt-1,ut, obtained by first-order Taylor expansion around the previous state xt-1.

fxt-1,ut=xt-1+∆t(vt-1cosφt-1)yt-1+∆t(vt-1sinφt-1)φt-1+∆tωtvt-1+∆v      (15)

Where ∆t is the time step, the yaw rate ωt [deg/sec] is measured by the IMU and is used as the control input of the system ut=ωt. The speed vt-1+∆v [m/sec] is obtained directly from the wheel encoders.

The system receives sensor data (camera) zt=[xaruco,yaruco, φaruco]T. This data is used in the update step to refine the state estimate xt and the covariance matrix Pt. First, we compute the innovation dt, where Ht=[I3x3 03x1]. Innovation represents the difference between the actual measurement and the predicted measurement. A small dt suggests good sensor-model agreement, while a large one may indicate noise or model error. However, its significance depends on what is considered a normal range; therefore, it is necessary to compute the residual covariance matrix St. It represents the uncertainty of the innovation dt​, combining both the predicted state uncertainty P and the uncertainty from the sensor R. The Kalman Gain Kt|t-1 acts as a weighting factor that determines how much to trust the new measurement compared to the model prediction. Finally, we update the estimated state and the covariance matrix.

xt=xt|t-1+Ktdt, Pt=[Ι-Ktdt]Pt|t-1-1    (16)

The updated state always lies between the prediction and the measurement, weighted by the confidence in each. After updating the covariance matrix, the uncertainty of the state decreases.

2.5.2 AEKF

In addition to EKF, we also experimented with the AEKF filter [9], which updates the process and measures noise matrices dynamically rather than using fixed values. Following the method in [10], we compute the residual εt=zt-Htxt. This represents the error between the actual measurement and the inferred measurement from the updated state. Therefore, the process noise and measurement noise matrices are adaptively calculated during the update phase.

Qt=αQQt-1+(1-αQ)(KtdtdtTKt)         (17)

Rt=αRRt-1+(1-αR)(εtεtT+HtPt|t-1HtT)  (18)

2.5.3 EKF with χ2-testing

In the EKF with χ2-testing [5], the chi-squared test is used to detect faulty sensor measurements. The χ2 value is computed from the innovation error, normalized by the innovation covariance matrix, as follows:

χ2=dtTSt-1dt, χ2≤threshold           (19)

If the χ2 value exceeds an experimentally defined threshold, the measurement is rejected as an outlier and skipped in the EKF update. Otherwise, correction proceeds as usual. This safeguards the localization system from major errors, ensuring stability and accuracy in noisy conditions.

- Results and Discussion

Table 2. Configuration parameters for EKF family

| Parameters | Values |
| --- | --- |
| threshold | 0.03 |
| Qt0 | [0.2, 0.2, 0.1, 0.02] |
| Rt0 | [0.005, 0.005, 0.001] |
| Pt0 | [0.1, 0.1, 0.1, 0.1] |

To evaluate trajectory tracking performance, we compare the robot’s estimated and ground-truth values of x, y, and yaw angle across different algorithms. As the Backstepping controller depends on state estimates, we also assess how estimation accuracy influences control performance by analyzing both estimated and ground-truth positions on the global map.

The reference trajectory is a circular path with a 1-meter radius. Localization is achieved using 12 ArUco markers placed at known positions (xaruco, yaruco, φaruco). The selected process and measurement noise covariance matrices Qt, Rt (shown in Table 3) are applied to all methods. For EKF and EKF with χ² threshold, these matrices remain constant Qt = Qt0 and Rt = Rt0, while AEKF dynamically adjusts them using scaling factors	 αQ and αR. The best configuration for AEKF was found to be {αQ, αR} = {0.95, 1}. The χ² threshold was set to 0.03 for the corresponding method.

Figure 7. EKF path and position error

Figures 7–9 show trajectory tracking results: ground-truth positions from ArUco markers (green), estimated positions (blue), and the reference trajectory (red). In Figure 7, the EKF enables the robot to follow the reference path but suffers from notable estimation errors, particularly in yaw, due to sensor noise.

Figure 8. AEKF path and position error

Figure 8 illustrates the benefits of adaptive covariance tuning in AEKF. The estimated trajectory is smoother and more closely aligned with the reference, indicating better noise handling and estimation accuracy. Figure 9 shows that EKF with χ² thresholding also performs well under uncertainty, similar to AEKF.

Figure 9. EKF with χ² threshold path and position error

Figure 10 highlights the role of an appropriate χ² threshold in rejecting outliers. Initially, floor noise caused large deviations, but once calibrated, the method produced accurate tracking. This confirms that the χ²-based outlier rejection effectively enhances localization in noisy environments.

Figure 10. χ² value and χ² threshold

To quantify the performance of each estimation method, we calculated the error between the estimated and ground-truth values of the robot’s position (x,y) and orientation θ at each time step. The ground truth is derived from the known global positions of ArUco markers, while the estimated values come from the respective filters. Let (xi​, yi​, θi) be the ground-truth pose at timestamp i, and (xi, yi, θi) be the corresponding estimated pose. The instantaneous errors are computed as:

exi= xi- xi, eyi= yi- yi, eθi= θi- θi

The mean error is the average of the absolute errors over the full trajectory:

Mean error in x= 1N i=1Nex​(i), similar for y and θ.

The peak error is defined as the maximum value among all instantaneous errors:

Peak error in  x = maxiex​(i), similar for y and θ.

Table 3. Peak localization errors for each filter

| Filter | Time(s) | Max error x (m) | Max error y (m) | Max error θ (rad) |
| --- | --- | --- | --- | --- |
| EKF | 98 | 0.212 | 0.245 | 0.167 |
| AEKF | 96 | 0.094 | 0.089 | 0.074 |
| EKF with χ² threshold | 92 | 0.142 | 0.137 | 0.092 |

Table 4. Mean localization errors of each filter

| Filter | Time(s) | Mean error x (m) | Mean error y (m) | Mean error θ (rad) |
| --- | --- | --- | --- | --- |
| EKF | 98 | 0.12 | 0.134 | 0.167 |
| AEKF | 96 | 0.047 | 0.043 | 0.053 |
| EKF with χ² threshold | 92 | 0.076 | 0.071 | 0.062 |

4. Conclusions

This paper presents a low-cost, comprehensive indoor localization system for ADAS-scale vehicles in pre-mapped environments. The system integrates ground truth measurement, multi-sensor fusion-based pose estimation, and control output. An enhanced Extended Kalman Filter (EKF) with adaptive noise tuning and Chi-square-based outlier rejection improves robustness in noisy conditions. A backstepping controller was used to assess state estimation accuracy under real-world control. Experimental results show that the system effectively filtered erroneous ArUco measurements and maintained reliable localization, demonstrating strong potential for indoor autonomous vehicle applications where GPS is unavailable.

Acknowledgement: This research is funded by Ho Chi Minh City University of Technology – VNU-HCM under grant number SVKSTN-2024-ĐĐT-12. We acknowledge the support of time and facilities from Ho Chi Minh City University of Technology (HCMUT), VNU-HCM for this study.

References

[1] J. Borenstein and Y. Koren, "Real-time obstacle avoidance for fast mobile robots," IEEE Transactions on Systems, Man, and Cybernetics, vol. 19, no. 5, pp. 1179–1187, 1989.

[2] Z. Chen, Y. Li, and Q. Zhang, "An Indoor Ultrasonic Positioning System Based on Mobile Robot," IEEE Sensors Journal, vol. 21, no. 3, pp. 3447–3457, 2021.

[3]	Z. Hendzel and L. Rykała, "Modelling of dynamics of a wheeled mobile robot with Mecanum wheels with the use of Lagrange equations of the second kind," Int. J. Appl. Mech. Eng., vol. 22, no. 1, pp. 81–99, 2017.

[4]	H. H. Giáp and N. Q. Đại, "Nghiên cứu thiết kế bộ điều khiển Backstepping ứng dụng cho rô bốt di động bốn bánh Mecanum," J. Sci. Technol., vol. 59, no. 3, pp. 31–37, Jun. 2023.

[5] B. Brumback and M. Srinath, "A chi-square test for fault-detection in Kalman filters," IEEE Trans. Autom. Control, vol. 32, no. 6, pp. 552–554, 1987.

[6] OpenCV, "Calibration with ArUco and ChArUco." [Online]. [Accessed: May. 4, 2025].  Available: https://docs.opencv.org/4.x/da/d13/tutorial_aruco_calibration.html

[7] "Low-pass filter design with microcontroller." [Online]. [Accessed: May. 11, 2025].  Available: https://www.researchgate.net/publication/365108086_Low-Pass_Filter_Design_with_Microcontroller

[8]  D. Schramm, M. Hiller, and R. Bardini, Single Track Models. Berlin, Heidelberg: Springer, 2014, pp. 223–253.

[9] S. Akhlaghi, N. Zhou, and Z. Huang, "Adaptive adjustment of noise covariance in Kalman filter for dynamic state estimation," in Proc. IEEE Power Energy Soc. Gen. Meeting, 2017.

[10]	 A. H. Mohamed and K. P. Schwarz, "Adaptive Kalman filtering for INS/GPS," J. Geodesy, vol. 73, pp. 193–203, 1999.